Part 4: Arduino Touch Sensor

Introduction:

In this activity you will construct a circuit and program the Arduino to connect the touch sensor and a single LED. The goal is to have the LED flash when you place your finger on the touch sensor.

Required materials:

- laptop with USB port

- Arduino Uno

- USB 2.0 cable, type A/B

- LED

- 2 220 ohm resistors

- mini breadboard

- touch sensor

- 4 jumper wires

Setup of circuit:

Connecting the touch sensor and LED through the Arduino:

- Once you have the circuit built, copy the following code into a new sketch.

// Capacitive Touch Sensor

// When Sig Output is high, touch sensor is being pressed

#define ctsPin 2   // Pin for capacitive touch sensor

int ledPin = 13;   // pin for the LED

void setup()

{

Serial.begin(9600);

pinMode(ledPin, OUTPUT);

pinMode(ctsPin, INPUT);

}

void loop()

{

int ctsValue = digitalRead(ctsPin);

if (ctsValue == HIGH)

{

digitalWrite(ledPin, HIGH);

Serial.println("TOUCHED");

}

else{

digitalWrite(ledPin,LOW);

Serial.println("not touched");

}

delay(500);

}

- Compile and download to the Arduino.

- Click on the magnifying glass in the upper-right corner to open the Serial Monitor.

- Press the touch sensor again, and the message in the Serial Monitor on the bottom of the screen should change from “Not Touched” to “Touched.”

- Show your teacher and get checked off for successfully completing this task.

- Name your sketch TouchSensor and save it to your desktop folder.