Pre-Activity Problem Set Answer Key

## Answer the following questions.

- What is the chemical equation for complete combustion?

Fuel + O2  CO2 + H2O

- What pollutants might result if incomplete combustion is occurring?

CO, VOCs, particulate matter (and still CO2 and H2O)

- Is NOx formed from the fuel itself? If not, where does it come from?	Yes      or     No

Thermal NOx is formed form the N2 already present in the air (air ~ 21% O2 and 79% N2).

- Which fuel has a higher energy content? 	Gasoline          or             Diesel

Answer the following questions using the example data in the table below.

| Independent Variables |  |  | Dependent Variables (concentrations observed at tailpipe) |  |  |
| --- | --- | --- | --- | --- | --- |
| Fuel | Formula | Combustion temperature (°C) | CO2 (ppm) | VOC (ppm) | NOx (ppm) |
| Case 1: Gasoline | C8H18 | 1500 | 3000 | 30 | 20 |
| Case 2: Diesel | C12H23 | 1900 | 3000 | 50 | 40 |
| Case 3: Ethanol | C2H5OH | 1500 | 4000 | 10 | 20 |

- What is similar about the chemical formulas for all of the fuel sources?

All contain carbon and hydrogen.

- What is different about ethanol, and why might that difference result in lower VOCs?

Ethanol has oxygen in the fuel, which facilitates more complete combustion and therefore fewer VOCs.

- Which requires more oxygen to reach complete combustion? Gasoline or diesel? And, why?

Diesel, because it contains more carbon, or a higher carbon/hydrogen ratio; you need two oxygens for each carbon to get to CO2.

- Which fuel type results in the most NOx and why?

Diesel, because of the higher combustion temperature.

- In the following table, circle the A or B that best completes the row. Then explain your rationale.

| Claim | Evidence | Reason |
| --- | --- | --- |
| Vehicle 1 produces more total pollutants than vehicle 2. | We observe higher CO2 and higher VOCs in the vehicle 1 data. | - Vehicle 1 has an older engine. - Vehicle 1 has a larger engine. |
| Vehicle 2 exhibits more complete combustion than vehicle 1. | - We observe more VOCs from vehicle 1 than vehicle 2. - We observe more VOCs from vehicle 2 than vehicle 1. | Vehicle 2 is newer and operating more efficiently, therefore it is displaying more complete combustion. |
| - The combustion in vehicle 1 is hotter. - The combustion in vehicle 2 is hotter. | We observe more NOx from vehicle 1 than vehicle 2. | Vehicle 1 has a diesel engine. |

Explanations

Row 1: A larger engine uses more fuel, which results in more overall emissions, while an older engine may run poorly and may exhibit more incomplete combustion, but not necessarily more emissions.

Row 2: More incomplete combustion leads to more VOCs.

Row 3: Diesel engines run at a hotter temperature and typically produce more NOx.

CO2

CO2

CHALLENGE QUESTION

- To the air-fuel ratio plot on the right, add a line for CO2.

CO2

CO2

Explanation of CO2 line placement (the red line). Moving from left to right:

- To begin with, CO2 will be at a minimum.

- It will increase as more O2 is added.

- It will plateau as CO reaches a minimum (and achieves more complete combustion).

- As the VOCs increase, the CO2 decreases slightly.