Tools and Equipment, Part II Activity –

Pulley Worksheet

Instruction/Questions

- What is the measured weight of your object? Load = ___________ - How is the Mechanical Advantage of a pulley system calculated? - Set up your pulley, weight and rope as shown at right. - What is the theoretical Mechanical Advantage of this system? MA = ___________ - What is the required force to raise object higher? Effort = ___________ - Calculate the actual Mechanical Advantage of the pulley system using the following equation: - How does this compare to the theoretical MA from above? - Support your load according to the pulley setup shown at right. - What is the theoretical Mechanical Advantage of this system? MA = ___________ - What is the required force to raise object higher? Effort = ___________ - Calculate the actual Mechanical Advantage of the pulley system using the following equation: - How does this compare to the theoretical MA from above? - Add another pulley to your system as shown to the right. - What is the theoretical Mechanical Advantage of this system? MA = ___________ - What is the required force to raise object higher? Effort = ___________ - Calculate the actual Mechanical Advantage of the pulley system using the following equation: - How does this compare to the theoretical MA from above?

Results

In general, were the theoretical mechanical advantages similar to the actual ones? ______________________________________________________________________________________________________________________________________________________________________

Discussion

- What were some sources of error in your experimental procedure? ________________________________________________________________________________________________________________________________________________________________ - What recommendations would you make as engineers trying to move the gray whale back to the ocean? Would you use pulleys? Why or why not? ________________________________________________________________________________________________________________________________________________________________ ________________________________________________________________________________ - What are some constraints that you as engineers might consider while designing a pulley system for the whale? ________________________________________________________________________________________________________________________________________________________________ - What impacts to the whale might you need to consider for moving it back to the ocean? ________________________________________________________________________________________________________________________________________________________________ - Calculate the percent error in the mechanical advantage of the actual pulleys compared to the theoretical pulleys. % Error= Theoretical Value-Actual ValueActual Value*100 Pulley Setup A: Pulley Setup B: Pulley Setup C: