Weather & Atmosphere, Lesson 6:

Global Climate Change – Carbon Footprint Worksheet

Instructions

Your “Carbon Footprint” is a calculation that tells you how much carbon dioxide you produce. Since carbon dioxide is a greenhouse gas, this worksheet can help you see how you contribute to global warming.

Family Survey

Answer each question for a yearly total (multiply a monthly total by 12). - Number of miles driven per year ___________ X (multiplied by) the vehicle’s average gas mileage ______ = ______ gallons of gasoline used. (Do this for each vehicle your family owns) _________________ X _________ = ___________ _________________ X _________ = ___________ _________________ X _________ = ___________ Total ____________ - Number of miles of air travel for your family _______________ - Average number of miles your family travels by bus/other mass transit_____________ . - Kilowatt-hours of electricity used _____________________ . (Look at your utility bill to find out a monthly amount and multiply by 12.) - Therms of natural gas used ________________________ . (Look at your utility bill to find out a monthly amount and multiply by 12.) - Gallons of propane or bottled gas used ____________________ . - Other sources of energy your family uses? ________________________________________________________________________________________________________________________________________________________________________________________________________________________ - Total number of people living in your household __________ .

Your “Footprint” on the Global Environment – The Greenhouse Effect

Use the information you gathered in your Family Survey to calculate how much CO2 (Carbon Dioxide) you generate per year. Carbon Dioxide is one of the main gases involved in creating the global Greenhouse Effect. 1.Gallons of gasoline used ________ x 22 pounds CO2/gallon = ________________ . 2.Miles of air travel ________ x 0.9 pounds/mile = ________________ . 3.Miles on mass transit ________ x 0.5 pounds/mile = ________________ . 4.Kilowatt hours ________ x 1.5 pounds/ kWh = ________________ . 5.Therms natural gas ________ x 11 pounds/therm = ________________ . 7.Gallons of propane ________ x 13 pounds/gallon = ________________ . Total pounds of CO2 generated (add up all the numbers on the right) ________________ . Total pounds generated (divided by) number of people in your household = __________________ (Total You Generated)

Reducing Your Impact

In the space below, write down some actions that you and your family could do to reduce the Greenhouse Effect. _________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________