Geiger Counter Arduino Sensors Code

This code works for Uno and Mega.

#include <Wire.h>

#include <SPI.h>

#include <LiquidCrystal_I2C.h>

#define LOG_PERIOD 15000  //Logging period in milliseconds, recommended value 15000-60000.

#define MAX_PERIOD 60000  //Maximum logging period without modifying this sketch

#define PERIOD 60000.0 // (60 sec) one minute measure period

volatile unsigned long CNT; // variable for counting interrupts from dosimeter

unsigned long counts;     //variable for GM Tube events

unsigned long cpm;        //variable for CPM

unsigned int multiplier;  //variable for calculation CPM in this sketch

unsigned long previousMillis;  //variable for time measurement

unsigned long dispPeriod; // variable for measuring time

unsigned long CPM; // variable for measuring CPM

// initialize the library with the numbers of the interface pins

LiquidCrystal_I2C lcd(0x27,16,2);  // set the LCD address to 0x27 for a 16 chars and 2 line display

void setup() { // setup

lcd.init();

lcd.init();

lcd.backlight();

CNT = 0;

CPM = 0;

dispPeriod = 0;

lcd.setCursor(0,0);

lcd.print(" Geiger Counter ");

delay(2000);

cleanDisplay();

attachInterrupt(0,GetEvent,FALLING); // Event on pin 2

}

void loop() {

lcd.setCursor(0,0); // print text and CNT on the LCD

lcd.print("CPM:");

lcd.setCursor(0,1);

lcd.print("CNT:");

lcd.setCursor(5,1);

lcd.print(CNT);

if (millis() >=dispPeriod + PERIOD) { // If one minute is over

cleanDisplay(); // Clear LCD

// Do something about accumulated CNT events....

lcd.setCursor(5, 0);

CPM = CNT;

lcd.print(CPM); //Display CPM

CNT = 0;

dispPeriod = millis();

}

}

void GetEvent(){ // Get Event from Device

CNT++;

}

void cleanDisplay (){ // Clear LCD routine

lcd.clear();

lcd.setCursor(0,0);

lcd.setCursor(0,0);

}