Activity Pre-Quiz Answer Key

The problem: A 25-m diameter semispherical segmented dome is going to be constructed (see Figure 1). Six equal-length segments will be used to create the revolving line (see Figure 2). Table 1 shows the relative positions of the revolving line vertices. Assuming the dome is a solid of revolution, find the dome’s volume. Show your work and give the result with three decimal places.

Figure 1

| x (m) | y (m) |
| --- | --- |
| 0.00000 | 5.00000 |
| 1.29410 | 4.82963 |
| 2.50000 | 4.33013 |
| 3.53553 | 3.53553 |
| 4.33013 | 2.50000 |
| 4.82963 | 1.29410 |

Table 1

Figure 2

Answer:

The same solid is obtained revolving around the x-axis or y-axis. The volume obtained revolving the segmented line around the y-axis can be computed the using the following formula:

Because no algebraic expression exists for the segmented line, a numerical integration must be performed. The trapezoidal rule for non-uniform partitions is appropriate for this problem:

Then, the volume can be computed using the following expression:

V  ½  [(52 + 4.829632)(1.2941 – 0) + (4.829632 + 4.330132)(2.5 – 1.2941) +

(4.330132 + 3.535532)(3.53553 – 2.5) + (3.535532 + 2.52)(4.33013 – 3.53553) +

(2.52 + 1.29412)(4.82963 – 4.33013)]

Taking R(yi) = xi, the x-values in Table 1:

V  258.843 m3

Note: Volume of half-sphere of radius 5 m: