Data Sensing Glove Lab

This lab will allow you to create a data sensing glove and test it out while measuring the electrical resistance in the strain sensors.

You will need to gather the materials for the lab:

- Two 3D printed sensors made from Conductive PLA

- Two 3D printed sensors made from ETPU

- One strand of wire that is 2 feet long

- Superglue

- Two gloves

- Scissors

- Sharpie

- 3D printed prosthetic hand

- Digital multimeter

The first step will be to construct the strain sensors.

- Cut the length of wire into eight segments.

- Strip about an inch off each end of every wire segment.

- Use the soldering iron to melt the exposed end of the wire into the end of one of the 3D printed sensors.

- Repeat this for the other end of the sensor.

- Repeat this method for the other three sensors.

The next step will be to construct the data sensing glove.

- Cut the glove about an inch below the thumb all the way across.

- Attach a conductive PLA sensor to the index finger on the rubberized side of the glove in the spot shown in the picture using superglue.

- Attach the second conductive PLA sensor to the middle finger on the rubberized side of the glove in the spot shown in the picture using superglue.

- Label this glove with the Sharpie as “PLA”.

- Make the second glove in the same manner using the ETPU sensors and label the glove as “ETPU”.

The next step will be to construct the experimental setup.

- Put the glove on the 3D printed prosthetic hand with the rubberized side (the side with the sensors) on the back of the hand.

- Attach the digital multimeter leads to the wires so that a completed circuit is made.

- The electrical resistance will need to be recorded when the sensor is at 0° of bending, 45° of bending, and 90° of bending. This needs to be recorded in the data tables that are provided.

- This will be need to be repeated two more times for the same glove.

- This method will be repeated for the other sensor on the glove.

- This method will be repeated for the other glove.

- See the Data Collection table on the next page.

After the data has been collected, you will need to graph it using graph paper or Excel.

|  | Electrical Resistance at 0° of bending (Ohms) | Electrical Resistance at 45° of bending (Ohms) | Electrical Resistance at 90° of bending (Ohms) |
| --- | --- | --- | --- |
| Conductive PLA Sensor One Trial One |  |  |  |
| Conductive PLA Sensor One Trial Two |  |  |  |
| Conductive PLA Sensor One Trial Three |  |  |  |
| Conductive PLA Sensor Two Trial One |  |  |  |
| Conductive PLA Sensor Two Trial Two |  |  |  |
| Conductive PLA Sensor Two Trial Three |  |  |  |
| ETPU Sensor One Trial One |  |  |  |
| ETPU Sensor One Trial Two |  |  |  |
| ETPU Sensor One Trial Three |  |  |  |
| ETPU Sensor Two Trial One |  |  |  |
| ETPU Sensor Two Trial Two |  |  |  |
| ETPU Sensor Two Trial Three |  |  |  |