Activity Worksheet

Part 2: Understanding the Arduino Microcontroller

Instructions: Implement a code to turn the LED on for 300 ms, off for 100 ms, on for 500 ms, and then off for 500 ms.

Part 3: Understanding the MAX30102 Spectrometer

Instructions: After watching this video, https://www.youtube.com/watch?v=MHPgamGQmDY (7:36 minutes), explain the importance of red light, infrared light, and the optical sensors to measure blood oxygen rate in MAX30102.

Part 4: Understanding the AD8232 Sensor

Instructions: First, view the video on Slide 15, https://www.youtube.com/watch?v=01y_Vu_sAQU&t=463s

(8:59 minutes), which shows how to connect the Arduino to the AD8232 sensor. Then:

- Connect the AD8323 sensor to the Arduino as in the video.

- Copy and paste the program in Slide 18.

- Compile and run.

- Choose serial plotter to capture ECG.

- Place the leads on the right position as in the video.

- Compute the peaks for 15 seconds.

- Save or take screenshots of heart rate and oxygen level readings.

Data Collection

Instructions: Collect data from the sensors.

- Test the MAX30102 and AD8323 on three students.

- For each student tested, write down the oxygen level and heart rate (HR) using MAX30102.

- Use serial plotter for AD8323 to calculate the (HR) = 4 * number of the peak per 15 seconds.

- Complete the data table below.

| Student Name | Oxygen Level Using MAX30102 | HR Using MAX30102 | Calculated HR Using AD8323 | Arrhythmia Tachy or Brady | Reason for Arrhythmia Case |
| --- | --- | --- | --- | --- | --- |

Reflection Questions

- Think about and write down the reasons for getting different HR measurements for the same student. Which one is more accurate?

- How could you improve the design to get more accurate results?

Optional: Application of Computational Thinking:

- How did you apply decomposition, pattern recognition, abstraction, and algorithm design in this activity?

- Can you think of other real-world applications where similar data analysis techniques might be useful?