Table 1. Titanium and Its Alloys

|  | Description | Properties | Uses |
| --- | --- | --- | --- |
| Titanium (Ti) | - Element - Atomic number: 22 - Common; ninth most abundant element in the Earth’s crust - Not harmful to living things | - Dark gray, shiny metal - Melting point: 1,677 oC - Density is 4.6 g/cm3 - Malleable and ductile at higher temperatures - Brittle at colder temperatures - Unreactive at room temperature - Non-corrosive - Does not oxidize at room temperature | - Making alloys - Sometimes used in jewelry - Sometimes used in bike frames |
| Ti Beta-C | - Alloy - Mostly composed of titanium, chromium and vanadium - Strong and durable | - Melting point: 1,760 oC - Density is 4.8 g/cm3 - Ductile at higher temperatures (1600 oC +) - Non-corrosive if oxide layer is present - Oxidizes at high temperatures | - Aircraft and race car springs - Underground tubes and pipes - Casing equipment for gas and oil wells (such as an enclosed pipe) - Fasteners (such as nuts and bolts) |
| 6al-4v | - Alloy - Mostly composed of titanium, aluminum and vanadium - Most-commonly used titanium alloy - Can be used in environments at temperatures of up to 350 oC - High strength and lightweight - When heated to approximately 430 oC, strength is weakened | - Melting point: 1660 oC - Density is 4.43 g/cm3 - Non-oxidizing - Non-corrosive - Reactive with hydrogen - Ductile at high temperatures (1600 oC +) | - Aircraft turbine engine components - Aircraft structural components - Aerospace fasteners - High-performance automotive parts - Marine applications - Medical devices - Sports equipment |