# Angular Velocity Experiment Worksheet

Figure 1. Example data logging experimental setup.

# Vocabulary/Definitions

| Word | Definition |
| --- | --- |
| data logging | Using a computer to collect and record data over an extended period of time through the use of a sensor. |
| length | The measure of how long something is from one end to another. |
| period | The length of time to complete one cycle. |
| gear | A rotating machine with cut teeth. |
| gear train | A set of gears that is able to transfer rotational motion. |
| circumference | The distance around a circle. |
| degree | The unit of measure for a given angle. |
| radian | The unit of measure for a given angle equal to 57.296 degrees. The measure of a circle is equal to 2π radians = 360°. |
| velocity | The distance traveled per unit time. |
| angular velocity | The change in rotation traveled per unit time. |
| optimal | Best suited for the situation. The best choice, arrangement, result or decision obtainable that meets specific requirements. |

# Hypothesis

# Materials

- LEGO MINDSTORMS Education NXT base set

- 2 LEGO MINDSTORMS NXT intelligent bricks

- calculator

- ruler

- 2 pieces electrical tape (not black)

To share with the entire class:

- computer with LEGO MINDSTORMS Education NXT Software 2.1 
(programming and data logging)

# Procedure

# Explain and/or sketch your data logging experimental setup.
List the steps you followed to construct the device.

# Data

# Calculate the angular velocity with the three physics-based equations:

- circumference = 2* *(radius)				(Equation 1)

- degrees = radians * (180/)				(Equation 2)

- angular velocity = degrees/ (time for one period)	(Equation 3)

| Gear Length | Circumference (radians) | Rotations (degrees) | Period (seconds) | Angular Velocity (degrees/second) |
| --- | --- | --- | --- | --- |
| big:big |  |  |  |  |
| medium:medium |  |  |  |  |
| small:small |  |  |  |  |
| big:medium |  |  |  |  |
| big:small |  |  |  |  |
| small:medium |  |  |  |  |

Now, let’s look at length and gear ratio:

| Gear Length | Circumference (radians) | Rotations (degrees) | Period (seconds) | Angular Velocity (Degrees/ seconds) |
| --- | --- | --- | --- | --- |
| 4 big |  |  |  |  |
| 4 medium |  |  |  |  |
| 4 small |  |  |  |  |
| 1 big:3 medium |  |  |  |  |
| 3 big:1 medium |  |  |  |  |
| 3 big:1 small |  |  |  |  |
| 1 big:3 small |  |  |  |  |
| 3 small:1 medium |  |  |  |  |
| 1 small:3 medium |  |  |  |  |

# Results

# What does your data tell you about gears and angular velocity?

# Conclusions

# Do your experimental results agree with your hypothesis? Why or why not?

# Optimal Solution

# What gear train is optimal for a high angular velocity?