Pre-Activity Worksheet Answer Key

A robot called Nemo has 2.5-inch wheels and a robot named Wall-E has 3.5-inch wheels. The motors of both robots have the same RPM (revolutions per minute). Both robots drive forward for 1 minute.

- Which of the following is true?

  - Wall-E will go farther than Nemo.

  - Nemo will go farther than Wall-E.

  - Both travel the same distance.

- Which robot will have a greater linear velocity?

  - Wall-E

  - Nemo

  - Both have the same linear velocity.

- Which robot will have a greater angular velocity?

  - Wall-E

  - Nemo

  - Both have the same angular velocity.

For the following questions, the robots have the same wheel diameter as above (2.5-in for Nemo and 3.5-in for Wall-E).

- Wall-E is driving with an angular velocity of 20 radians/sec. Calculate Wall-E’s linear velocity.

v=ωr		ω=20 radsec	r=3.52in

v=20 rad1 sec⋅1.75 in=35 insec

- Nemo is driving with an angular velocity of 20 radians/sec. Calculate Nemo’s linear velocity.

v=ωr		ω=20 radsec	r=2.52in

v=20 rad1 sec⋅1.25 in=25 insec

- You want Wall-E and Nemo to end up at the same location. Given their respective wheel sizes, describe how Wall-E and Nemo can drive to end up at the same location.

Wall-E could drive for a shorter time than Nemo drives. Wall-E will get there first, but they will end up in the same location. -OR- If Wall-E drives with fewer RPM (or a smaller angular velocity) than NEMO does, then they arrive at the same location and at the same time.