Alternative fuel: Ethanol

Instructions: Read this webpage (https://afdc.energy.gov/vehicles/flexible_fuel_emissions.html) and then answer the questions below.

Ethanol Vehicle Emissions:

How is ethanol used?  By itself or mixed with other substances?

Life Cycle Emissions:

What is a life cycle analysis?

What are the greenhouse gas (GHG) emissions?

How can the carbon dioxide emission be offset?

How does the emission with ethanol compare to the emission with gasoline?

Evaporative and Tailpipe Emissions:

What are the emissions of primary concern?

Use the GREET excel database to complete the chart below:

- Open this link:  https://greet.es.anl.gov/greet_1_series

- Click the link underneath “GREET 1 Series (Fuel-Cycle Model) or this link GREET_2020rev1.zip

- Open the GREET folder

- Select “GREET1-2020”

- To use the GREET database, you have to click on the tab at the bottom of the screen. Ethanol is abbreviated EtOH, so click the “EtOH” tab.  The red arrow above is pointing to it.

- There is a lot of information on this database.  Scroll all the way down to 4) Summary of Energy Consumption, Water Consumption, and Emissions.  The data you are looking for is listed in table 4.1.  This table tells you the energy consumption, water consumption, and total emissions for the use of ethanol fuel.

- Because we are interested in reducing carbon emissions and climate change, you will be looking at the values for methane (CH4), carbon dioxide (CO2), and nitrous oxide (N2O).  There are other variables in this chart, but we will focus just on these three.  There is a red box around them in the table below.

- There are many different plants that are used to make ethanol.  Look through the data table and find the type of ethanol that you think is best in regards to the amount of CO2, N2O, CH4 in the emissions.  To move through the data table, use the arrow that has the red circle around it in the picture above.  Record the data in the table below.  This is what you will share when the group comes back together.  (there are extra lines in the data table, you can use them if it is helpful to record information while trying to determine which version of ethanol you want to use - circle the one that you will share)

| Gas emission | Ethanol type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CH4 |  |  |  |  |  |  |
| N2O |  |  |  |  |  |  |
| CO2 |  |  |  |  |  |  |

The abbreviations in GREET are defined below:

VOC = volatile organic compounds

CO = carbon monoxide

NOX = nitric oxide

PM10 = particulate matter with a diameter of 10 micrometers or less

PM2.5 = particulate matter with a diameter of 2.3 micrometers or less

SOX = sulfur oxides

BC = black carbon (particulate matter/ soot & contributes to climate change)

OC = organic carbon (respiratory effects)

CH4 = methane

N2O = nitrous oxide

CO2 = carbon dioxide

- Fill in the row below for ethanol.

- When everyone is finished learning about the energy sources, share what you have learned with the group.  Each individual should summarize the questions they answered and share the GREET emissions that were calculated.  Notes should be taken in the table below so that the information can be shared with your poster group.

- Highlight the energy source you will use to heat your building (remember that we are assuming that the technology for this will be in place) and complete the information below the table.

| Energy Source | Information about energy source | GREET values |
| --- | --- | --- |
| Ethanol |  |  |
| Electric |  |  |
| Biodiesel |  |  |
| Natural Gas |  |  |
| Propane |  |  |
| Hydrogen |  |  |

Type of fuel that will be recommended for use in heating your building structure:

Evidence and reasoning for this recommendation:

- Return to the “Energy Source” document and continue to step 2.