Block Crusher Design Instructions

Materials:

2 x 6 x 8 inch pieces of lumber (2 pieces)

8 inch eyebolt, with washer and nut that fit

6 inch U-bolt with washers and nuts

16 inch long piece (or 2-8 inch pieces) of 1/4in steel

300 lb. crane scale

1 or 2 inch wood screws (box)

Tools:

Electric drill

Saws (for wood and metal)

Tape measure

Drill bits

Assembly Instructions:

- Cut the lumber:

  - 4 pieces, 14.5 inches long

  - 4 pieces, 6 inches long

  - 2 pieces, 24.5 inches long

  - 2 pieces, 17 inches long

- Assemble the frame:

Figure 1

Make a box that is 17 inches wide and 27.5 inches tall. (Figure 1)

Figure 1: The 17 inch pieces lay across the top and bottom. The 24.5 inch pieces are the sides of the frame. The 14.5 inch and 6 inch pieces are the internal supports.

Use the two 17 inch pieces at the top and bottom

    - Screw the 24.5 inch pieces into the sides of the box, so that they sit inside the 17 inch pieces.

- Add the supports

  - On the inside of the box, attach the other pieces of lumber. Start at the top of the box, and screw the top corners the 14.5 inch pieces; two of them on each side. (Figure 2a)

  - At the bottom of the box, attach the 6 inch pieces. (Figure 2b)

Figure 2a

Figure 2b

Figure 2a and 2b: Details of the internal support. Place two 14.5 inch pieces at the top running along each (inner) side and two 6 inch pieces at the bottom along each (inner) side. There should be a gap in the middle.

- Make the test rig.

  - Find the middle point of the top of the frame. Drill a hole just larger than your eye bolt; it will be attached up there (Figure 3a and 3b).

  - Cut the steel pieces into 2- 8inch pieces.

  - Measure the width of the U-bolt and drill holes into both pieces of the metal so that the U-bolt can slide into it.  (Figure 4a)

  - Attach the U bolt to the bottom of the crane scale and the eye bolt to the top; your machine is complete. (Figure 4b)  😊

Figure 3b

Figure 3a

Figure 3a: Drill a hole in the middle of the top piece of lumber for the eyebolt.

Figure 3b: Place the eyebolt into the hole; fasten with a washer and nut.

Figure 4a: Attach the two pieces of steel to the U-bolt; take care to drill the holes carefully. The nuts and washers of the U-bolt hold the steel in place.

Figure 4b: Congratulations! The finished product!

Figure 4b

Figure 4a