Myth/Fact Assessment Sheet Answer Key

| Pre-Test | Statement | Post-Test | What Did I Learn? |
| --- | --- | --- | --- |
| Myth/Fact | Semiconductors conduct electricity the same way metals do. | Myth/Fact | Semiconductors conduct electricity differently from metals. They only conduct under certain conditions, allowing precise control in electronics. |
| Myth/Fact | Photolithography is a process used to create tiny patterns on semiconductor surfaces. | Myth/Fact | Photolithography uses light to transfer patterns onto semiconductor materials, forming circuits and microchips. |
| Myth/Fact | A photoresist layer gets removed by UV light during photolithography. | Myth/Fact | UV light polymerizes or chemically changes the photoresist in certain areas, making them either harder or easier to remove in later steps. |
| Myth/Fact | Wavelength doesn’t matter when exposing a photoresist to light. | Myth/Fact | The wavelength of UV light is crucial because only certain wavelengths cause the photoresist to undergo polymerization effectively. |
| Myth/Fact | Masking controls which areas of a semiconductor are exposed to light. | Myth/Fact | A mask blocks UV light in some areas, allowing engineers to control which parts of the photoresist are polymerized during photolithography. |
| Myth/Fact | In manufacturing, exposure time has no effect on the resolution of a pattern surface. | Myth/Fact | Exposure time affects how much polymerization occurs, influencing the clarity and sharpness (resolution) of the patterns.. |
| Myth/Fact | Photolithography is only used in electronics and has no other applications. | Myth/Fact | Though semiconductor manufacturing is a key application, photolithography is also used in microfluidics, solar cells, and biomedical devices. |
| Myth/Fact | Polymerization is a chemical reaction where small molecules join to form a larger chain. | Myth/Fact | This is the correct definition of polymerization, a critical step in curing the photoresist when exposed to UV light. |
| Myth/Fact | The distance between the UV light and the masking pattern does not impact resolution. | Myth/Fact | The distance affects resolution because greater distances cause light to scatter or blur, reducing the sharpness of the pattern on the photoresist. |