Beyond Binary Quiz Answer Key

Instructions: Fill in the blanks with the logic gate type that is represented in the image. Your choices are AND, NOT, or OR.

- _____NOT_________

- _____AND_________

- _____OR_________

- _____OR_________

- ______AND________

- ______NOT________

- The following is a set of logic gates that create a smart control for a sprinkler system. Create a set of truth tables for each gate using words (not 1s and 0s), and then write a statement that explains all of the conditions that must be true for the sprinkler system to automatically turn on.

OR Gate

| Input A | Input B | Output C |
| --- | --- | --- |
| Sensor reads below acceptable soil moisture (high input) | Sensor reads high temperature (high input) | High (on) signal sent to AND gate |
| Sensor reads below acceptable soil moisture (high input) | Sensor reads low temperature (low input) | High (on) signal sent to AND gate |
| Sensor reads soil moisture levels in acceptable range (low input) | Sensor reads high temperature (high input) | High (on) signal sent to AND gate |
| Sensor reads soil moisture levels in acceptable range (low input) | Sensor reads low temperature (low input) | Low (off) signal sent to AND gate |

NOT Gate

| Input D | Output E |
| --- | --- |
| Sensor detects that it is raining (high input) | Low (off) signal sent to AND gate |
| Sensor does not detect rain (low input)) | High (on) signal sent to AND gate |

AND Gate

| Input C | Input E | Output F |
| --- | --- | --- |
| High signal means either high temp or dry soil | High signal means it is not raining | High (on) signal sent to turn the sprinkler system on |
| High signal means either high temp or dry soil | Low signal means it is raining | Low (off); sprinkler system remains off |
| Low signal means soil is both moist and cool | High signal means it is not raining | Low (off); sprinkler system remains off |
| Low signal means soil is both moist and cool | Low signal means it is raining | Low (off); sprinkler system remains off |

- Choose one of the two scenarios below. Sketch the logic gate integrated circuit described by the scenario. Then create truth tables using words, not 1s and 0s, to describe how each gate functions and integrates with the others to complete the task.

| Elevator Control: An elevator moves up when either the up button is pressed inside the elevator or the up button is pressed outside the elevator, but not when the emergency stop button has been pressed. | Secure Building Access: A user can gain access when both the correct username and password are entered or a valid biometric scan occurs, but not if the building is on lockdown. |
| --- | --- |

OR Gate

| Input A | Input B | Output D |
| --- | --- | --- |
| Sensor reads pushed button inside elevator (high input) | Sensor reads pushed button outside elevator (high input) | High (on) signal sent to AND gate |
| Sensor reads pushed button inside elevator (high input) | Sensor does not read pushed button outside elevator (low input) | High (on) signal sent to AND gate |
| Sensor does not read pushed button inside elevator (low input) | Sensor reads pushed button outside elevator (high input) | High (on) signal sent to AND gate |
| Sensor does not read pushed button inside elevator (low input) | Sensor does not read pushed button outside elevator (low input) | Low (off) signal sent to AND gate |

NOT gate

| Input C | Output E |
| --- | --- |
| Sensor detects that the emergency stop button has been pushed (high input) | Low (off) signal sent to AND gate |
| Sensor detects that the emergency stop button has not been pushed (low input) | High (on) signal sent to AND gate |

AND Gate

| Input D | Input E | Output F |
| --- | --- | --- |
| High (on) signal sent from elevator button push | Low (off) signal sent representing emergency stop is pushed | Low (off); elevator will not move |
| High (on) signal sent from elevator button push | High (on) signal sent representing emergency stop is not pushed | High (on); elevator set to move |
| Low (off) signal sent from elevator button | Low (off) signal sent representing emergency stop pushed | Low (off); elevator will not move |
| Low (off) signal sent from elevator button | High (on) signal sent representing emergency stop is not pushed | Low (off); elevator will not move |

AND Gate

| Input A | Input B | Output C |
| --- | --- | --- |
| High (on) signal sent representing correct username | Low (off) signal sent representing incorrect password | Low (off); Log-in information not confirmed |
| High (on) signal sent representing correct username | High (on) signal sent representing correct password | High (on); Log-in information confirmed |
| Low (off) signal sent representing incorrect username | Low (off) signal sent representing incorrect password | Low (off); Log-in information not confirmed |
| Low (off) signal sent representing incorrect username | High (on) signal sent representing correct password | Low (off); Log-in information not confirmed |

OR Gate

| Input C | Input D | Output F |
| --- | --- | --- |
| High (on); Log-in information confirmed | Bioscan confirms identity; high (on) | High (on) signal sent to AND gate |
| High (on); Log-in information confirmed | Bioscan denies identity; low (off) | High (on) signal sent to AND gate |
| Low (off); Log-in information not confirmed | Bioscan confirms identity; high (on) | High (on) signal sent to AND gate |
| Low (off); Log-in information not confirmed | Bioscan denies identity; low (off) | Low (off) signal sent to AND gate |

NOT gate

| Input E | Output G |
| --- | --- |
| Building is in lockdown; high (on) | Low (off) signal sent to AND gate |
| Building is not in lockdown; low (off) | High (on) signal sent to AND gate |

AND Gate

| Input F | Input G | Output H |
| --- | --- | --- |
| High (on) signal sent to AND gate from identification sensors | Low (off) signal sent to AND gate representing building is in lockdown | Low (off); Building access not granted |
| High (on) signal sent to AND gate from identification sensors | High (on) signal sent to AND gate representing building is not in lockdown | High (on); Building access granted |
| Low (off) signal sent to AND gate from identification sensors | Low (off) signal sent to AND gate representing building is in lockdown | Low (off); Building access not granted |
| Low (off) signal sent to AND gate from identification sensors | High (on) signal sent to AND gate representing building is not in lockdown | Low (off); Building access not granted |