Beyond Binary Worksheet Answer Key

Objective: To understand and select the appropriate logic gate based on a given scenario; build the gate; and combine it with others in your group to achieve the required functionality.

- What is your given scenario? Industry: Mechanical Press; Security; Agricultural: Smart Greenhouse Ventilation; Pet Door; Automotive Headlights; Industry: Chemical

- What is your role? Answers vary; see charts below.

- What is your first input? Answers vary; see charts below.

- What is your second input? Answers vary; see charts below.

- Complete the table below: See charts below.

Industry: Mechanical Press

Authorizing users through badges or master keys (OR Gate)

| Input A: Badge scan status | Input B: Master key status | Output C: Authorized user signal |
| --- | --- | --- |
| Badge is scanned successfully; signal HIGH | Master key engaged; signal HIGH | Both badge and key signals received; signal HIGH |
| Badge is scanned successfully; signal HIGH | Master key missing; signal LOW | At least one input present; signal HIGH |
| Badge is not scanned; signal LOW | Master key engaged; signal HIGH | At least one input present; signal HIGH |
| Badge is not scanned; signal LOW | Master key missing; signal LOW | Both badge and key signals are missing; signal LOW |

Users and safety shield verification (AND Gate)

| Input C: Authorized user signal | Input D: Safety shield location | Output E: User and shield status |
| --- | --- | --- |
| At least one form of ID is present; signal HIGH | Safety shield down; signal HIGH | User and shield are good to run; signal HIGH |
| At least one form of ID is present; signal HIGH | Safety shield up; signal LOW | User or shield status is not acceptable; signal LOW |
| Missing authorization for use; signal LOW | Safety shield down; signal HIGH | User or shield status is not acceptable; signal LOW |
| Missing authorization for use; signal LOW | Safety shield up; signal LOW | User or shield status is not acceptable; signal LOW |

Emergency Stop Button System (NOT)

| Input F: Emergency stop button status | Output G: Emergency stop system signal* |
| --- | --- |
| Emergency button is pressed; signal HIGH | System needs to stop ASAP; signal LOW |
| Emergency button is NOT pressed; signal LOW | System is not facing an emergency and should continue working; signal HIGH |

*Note: Output G is inverted to integrate with AND logic for final system operability.

Emergency Stop Button System (AND Gate)

| Input G: Emergency stop system signal | Input E: User and shield status | Output H: System operability |
| --- | --- | --- |
| System is not facing an emergency and should continue working; signal HIGH | User and shield are good to run; signal HIGH | System is cleared to run; signal HIGH |
| System is not facing an emergency and should continue working; signal HIGH | User or shield status is not acceptable; signal LOW | System fault is present; signal LOW |
| System needs to stop ASAP; signal LOW | User and shield are good to run; signal HIGH | System fault is present; signal LOW |
| System needs to stop ASAP; signal LOW | User or shield status is not acceptable; signal LOW | System fault is present; signal LOW |

Security System

Exterior Breach sensor (OR Gate)

| Input A: Door status | Input B: window status | Output C: Security breach detected signal |
| --- | --- | --- |
| Door is breached; signal HIGH | Window is breached; signal HIGH | Exterior breach trigger; signal HIGH |
| Door is breached; signal HIGH | Window is not breached; signal LOW | Exterior breach trigger; signal HIGH |
| Door is NOT breached; signal LOW | Window is breached; signal HIGH | Exterior breach trigger; signal HIGH |
| Door is NOT breached; signal LOW | Window is not breached; signal LOW | No exterior breach indicated; signal LOW |

Accidental triggering during business hours prevention (AND Gate)

| Input C: Security breach detected signal | Input D: Timing system signal | Output E: Security breach validated by time of day |
| --- | --- | --- |
| Exterior breach detected; signal HIGH | Outside of business hours; signal HIGH | Suspected unauthorized presence; signal HIGH |
| Exterior breach; signal HIGH | Inside of business hours; signal LOW | No suspected unauthorized presence; signal LOW |
| No exterior breach indicated; signal LOW | Outside of business hours; signal HIGH | No suspected unauthorized presence; signal LOW |
| No exterior breach indicated; signal LOW | Inside of business hours; signal LOW | No suspected unauthorized presence; signal LOW |

Disarming System/ Manual override code entered (NOT Gate)

| Input F: Manual disarming code use | Output G: Manual override code status |
| --- | --- |
| Manual disarming code correctly entered; signal HIGH | System override engaged; signal LOW |
| Manual disarming code not entered; signal LOW | No manual override; signal HIGH |

Security Alarm Activation and Sound (2nd AND Gate)

| Input E: Security breach validated by time of day | Input G: Manual override signal | Output H: Security alarm activates |
| --- | --- | --- |
| Suspected unauthorized presence; signal HIGH | System override engaged; signal LOW | System doesn’t activate or sound; signal LOW |
| No suspected unauthorized presence; signal LOW | System override engaged; signal LOW | System doesn’t activate or sound; signal LOW |
| Suspected unauthorized presence; signal HIGH | No manual override; signal HIGH | System activates or sounds; signal HIGH |
| No suspected unauthorized presence; signal LOW | No manual override; signal HIGH | System doesn’t activate or sound; signal LOW |

Smart Greenhouse Ventilation System

Humidity and Temperature Sensor (OR Gate)

| Input A: Interior humidity sensor | Input B: Temperature sensor in greenhouse | Output C: Signal for temp/humidity sensors |
| --- | --- | --- |
| High humidity in the greenhouse; signal HIGH | Excessive temperature in the greenhouse; signal HIGH | Humidity/temperature sensor; signal HIGH |
| Low humidity in the greenhouse; signal LOW | Excessive temperature in the greenhouse; signal HIGH | Humidity/temperature sensor; signal HIGH |
| High humidity in the greenhouse; signal HIGH | Acceptable temperature in the greenhouse; signal LOW | Humidity/temperature sensor; signal HIGH |
| Low humidity in the greenhouse; signal LOW | Acceptable temperature in the greenhouse; signal LOW | Humidity/temperature; signal LOW |

Outdoor Climate Sensor (NOT gate)

| Input D: Raining outside | Output E: Signal regarding rain |
| --- | --- |
| Rain detected; signal HIGH | Ventilation blocked by rain; signal LOW |
| No rain detected; signal LOW | Ventilation not limited by rain; signal HIGH |

System Integration Specialist (AND)

| Input C: Signal for humidity/temperature sensor | Input E: Signal regarding rain | Output F: Ventilation status |
| --- | --- | --- |
| Humidity/temperature sensor; signal HIGH | Ventilation not limited by rain; signal HIGH | Ventilation on; signal HIGH |
| Humidity/temperature sensor; signal HIGH | Ventilation blocked by rain; signal LOW | Ventilation off; signal LOW |
| Humidity/temperature sensor; signal LOW | Ventilation not limited by rain; signal HIGH | Ventilation off; signal LOW |
| Humidity/temperature sensor; signal LOW | Ventilation blocked by rain; signal LOW | Ventilation off; signal LOW |

Smart Pet Door System

Pet verification system (OR Gate)

| Input A: Interior proximity sensor | Input B: Exterior proximity sensor | Output C: Pet presence signal |
| --- | --- | --- |
| Interior sensor triggered; signal HIGH | Exterior sensor triggered; signal HIGH | Pet presence detected; signal HIGH |
| Interior sensor triggered; signal HIGH | Exterior sensor not triggered ; signal LOW | Pet presence detected; signal HIGH |
| Interior sensor not triggered; signal LOW | Exterior sensor triggered; signal HIGH | Pet presence detected; signal HIGH |
| Interior sensor not triggered ; signal LOW | Exterior sensor not triggered ; signal LOW | Pet presence not detected; signal LOW |

Door Swing Safety Control (AND Gate)

| Input D: Door swing clearance | Input C: Pet presence signal | Output E: Sensor signal for presence and swing clearance |
| --- | --- | --- |
| Door clearance is confirmed; signal HIGH | Pet presence verified; signal HIGH | Sensor signal for presence and swing clearance; signal HIGH |
| Door clearance is confirmed; signal HIGH | Pet presence not indicated; signal LOW | No sensor signal for presence and swing clearance; signal LOW |
| Door is blocked; signal LOW | Pet presence verified; signal HIGH | No sensor signal for presence and swing clearance; signal LOW |
| Door is blocked; signal LOW | Pet presence not indicated; signal LOW | No sensor signal for presence and swing clearance; signal LOW |

Security Mode for house alarm Sensor (NOT gate)

| Input F: Security Mode for house alarm | Output G: signal regarding security mode |
| --- | --- |
| Security mode engaged; signal HIGH | Signal for security mode override off; signal LOW |
| Security mode off; signal LOW | Signal for security mode override on; signal HIGH |

Reasons for opening door and security mode status (2nd AND Gate)

| Input E: Sensor signal for approving door opening | Input G: Signal regarding security mode | Output H: Pet door swings open |
| --- | --- | --- |
| Door opening is approved; signal HIGH | Signal for security mode override on; signal HIGH | Signal is sent for door to open; signal HIGH |
| Door opening is approved; signal HIGH | Signal for security mode override off; signal LOW | Signal is not sent for door to open; signal LOW |
| Door opening is not approved; signal LOW | Signal for security mode override on; signal HIGH | Signal is not sent for door to open; signal LOW |
| Door opening is not approved; signal LOW | Signal for security mode override; signal off LOW | Signal is not sent for door to open; signal LOW |

Smart Automatic Headlight Control System

Low light and rain sensors (OR Gate)

| Input A: Photocell reading | Input B: Windshield wiper status | Output C: Signal for wipers/photocell |
| --- | --- | --- |
| Photocell reads low light; signal HIGH | Windshield wipers on; signal HIGH | Signal for wipers /photocell; signal HIGH |
| Photocell reads low light; signal HIGH | Windshield wipers off; signal LOW | Signal for wipers /photocell; signal HIGH |
| Photocell does NOT read low light; signal LOW | Windshield wipers on; signal HIGH | Signal for wipers /photocell; signal HIGH |
| Photocell does NOT read low light; signal LOW | Windshield wipers off; signal LOW | Signal for wipers /photocell; signal LOW |

Ignition and power connection (AND Gate)

| Input D: Engine Status | Input C: Signal for wipers/photocell | Output E: Signal for engine; lights; and wipers |
| --- | --- | --- |
| Engine is on; signal HIGH | Signal for wipers/photocell is on; signal HIGH | Engine is on and wipers/photocell signal is on; signal HIGH |
| Engine is on; signal HIGH | Signal for wipers/photocell is off; signal LOW | Engine is on and wipers/photocell signal is off; signal LOW |
| Engine is off; signal LOW | Signal for wipers/photocell is on; signal HIGH | Engine is off and wipers/photocell signal is on; signal LOW |
| Engine is off; signal LOW | Signal for wipers/photocell is off; signal LOW | Engine off and wipers/photocell signal is off; signal LOW |

Manual override control (NOT Gate)

| Input F: Manual override status | Output G: Override signal |
| --- | --- |
| Manual override is on HIGH | Automatic control is blocked LOW |
| Manual Override is off LOW | Automatic control is on HIGH |

Ignition and power connection (AND Gate)

| Input E Signal for engine; lights; and wipers | Input G Override signal | Output H Automatic headlight status |
| --- | --- | --- |
| Signal for engine; lights; and wipers HIGH | Automatic control is on HIGH | Headlights on automatically HIGH |
| Signal for engine; lights; and wipers HIGH | Automatic control is blocked LOW | Headlights are not on automatically LOW |
| Engine; wipers/ photocell signal is LOW | Signal for wipers /photocell HIGH | Headlights are not on automatically LOW |
| Engine; wipers/ photocell signal is off LOW | Automatic control is on HIGH | Headlights are not on automatically LOW |

Smart Chemical Manufacturing System

Pressure and temperature sensors (AND Gate)

| Input A: Pressure limit sensor | Input B: Temperature limit sensor | Output C: Pressure/temperature out-of-range signal |
| --- | --- | --- |
| Pressure too high; signal HIGH | Temperature too high; signal HIGH | Both pressure and temperature are out-of-range; signal HIGH |
| Pressure too high; signal HIGH | Temperature normal; signal LOW | Pressure out of range but temperature within range; signal LOW |
| Pressure normal; signal LOW | Temperature too high; signal HIGH | Pressure within range but temperature out of range; signal LOW |
| Pressure normal; signal LOW | Temperature normal; signal LOW | Both pressure and temperature are within range; signal LOW |

Gas leak or Pressure/temperature detection (OR Gate)

| Input D: Gas leak detection | Input C: Pressure/ temperature out-of-range signal | Output E: Gas leak and pressure/temperature signal |
| --- | --- | --- |
| Gas leak detected; signal HIGH | Pressure/temperature signal in range; signal LOW | Gas leak but pressure/temperature signal is within range; signal HIGH |
| Gas leak detected; signal HIGH | Pressure/temperature signal out of range; signal HIGH | Gas leak and pressure/temperature signal is out of range; signal HIGH |
| Gas leak not detected; signal LOW | Pressure/temperature signal in range; signal LOW | No gas leak and pressure/temperature signal is within range; signal LOW |
| Gas leak not detected; signal LOW | Pressure/temperature signal out of range; signal HIGH | No gas leak but pressure/temperature signal is out of range; signal HIGH |

Shutdown initiation (NOT Gate)

| Input E: Signal for gas leak; pressure/temperature | Output F: System shutdown initiation |
| --- | --- |
| Gas, temperature or pressure warning signal is activated; signal HIGH | Machines shut down for safety; signal LOW |
| Gas, temperature or pressure warning signal not activated; signal LOW | Machines continue as normal; signal HIGH |

- Describe another scenario using your gate; detailing the inputs and outputs.

Answers will vary.

- Create a diagram using logic gate symbols to show how you combined your gates to solve your problem.

See the logic equations; diagrams and truth tables in the slides.

- Create truth tables for each gate to indicate how they work together to solve the problem.

See the appropriate section above and the logic equations; diagrams and truth tables in the slides.

- How did this experience impact your understanding of how multiple inputs can be used to solve problems? Why is it important?

Answers vary but should include some reference to how most decisions are based on more than one factor and the way answers are determined depends on how the factors are related; can also reference levels of abstraction used to understand computers.