Name:									Date:

Group members:							Class:

Estimating Storage Capacity Worksheet

- Calculate d

|  | - d is the spacing of the structure (here: track pitch) - θ is the angle of the mth diffracted ray - m is the order of the diffracted ray. Here we only use the first order, i.e. m=+1, -1 |
| --- | --- |

To get a better estimate for d, calculate the average  in the last column.

|  | Laser color | Wavelength (nm) | Θ, m=+1 | Θ,m=-1 | d, m=+1 | d, m=-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CD |  |  |  |  |  |  |  |
| DVD |  |  |  |  |  |  |  |

- Estimate the storage

Using your measured distance d between tracks, how many tracks fit on a disc if 33mm are writable?

A CD track has around 270,000 pits. A DVD track fits around 500,000 pits because the pits are smaller. How many pits fit on a CD and DVD?

Divide the number by 8 to get an estimate of the storage in bytes. Compare with the info on the CD/DVD. Note that the DVD has two layers.

- Blu-ray discs need special readers that rely on blue lasers. How can a blu-ray disc store more information? Why is the laser blue?

- What are two advantages of using digital media to store data? What is a disadvantage?