Measuring Surface Tension (Method 2)

Instructions: During this activity, you will be measuring the surface tension of water and water with a surfactant added to it. In the first part of the activity, you will be assembling the testing apparatus, which will consist of a triple beam balance, button, and thread. In the second part of the activity, you will be using a triple-beam balance to measure the surface tension of different solutions.

Part 1: Assembling the Testing Apparatus

| Materials |  |
| --- | --- |
| - 50 mL beaker - 1 spool of thread - 1 medium plastic button | - 1 triple-beam pan balance - 1 ruler with units in centimeters |

Set up:

- Using your ruler, measure the radius of your button and record it here___________.

- Now convert the radius measurement to meters. (Note: you will need this for calculations later) _______________.

- Using your thread, thread the button to create a single loop. Follow the threading diagram below:

Flat face of button

1

2

3

4

= in front of button

= behind button

- Once the button is correctly threaded, tie the two ends of the string so that you have a continuous loop suspending the button. It should look like this:

Knot

- Adjust the platform of your triple-beam balance to be above the weighing pan of your balance. It should look like this:

Platform

Weighing pan

Balance base

- Attach your threaded button to the bottom hook of your balance beam and the weighing pan to the top hook of your balance beam.

- Once assembled, your testing apparatus should look like this:

- Using the sliding weights on the beams, balance your apparatus. Note the starting weight of the testing apparatus here:________________

- Remaining assembly will be done right before conducting the trials.

Part 2: Conducting Surface Tension Trials

| Materials |  |
| --- | --- |
| - Assembled testing apparatus from Part 1 - 50 mL beaker - 50 mL water (preferably distilled) - 50 mL Chemical #1 | - 50 mL Chemical #2 - 50 mL Chemical #3 - 50 mL Chemical #4 |

- Place a 50 mL beaker on the platform of your assembled testing apparatus.

- Fill your beaker until the level of the liquid comes in contact with the bottom of the button. Record the volume of liquid added to the beaker here: _____________

- To test the surface tension of a liquid:

  - Start from the weight where the button is floating on the surface of the liquid and the apparatus is balanced.

  - Slowly adjust the weights on the balance until the button breaks away from the surface of the liquid.

  - Record the weight at which the button broke away in the data table below.

  - Repeat your measurement at least three times for each tested liquid.

  - When changing liquids, make sure you fill your beaker to the above noted volume and set the apparatus to the above noted weight to start.

- Once you have collected all of your data, complete the calculations and answer the questions.

Data Sheet

| Langmuir-Pockels Testing Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Liquid | Trial | Initial Weight (gf) | Weight at Surface Break (gf) | Weight Applied to Break (gf) |
| Water | 1 |  |  |  |
| Water | 2 |  |  |  |
| Water | 3 |  |  |  |

| Unknown Chemical #1 | 1 |  |  |  |
| --- | --- | --- | --- | --- |
| Unknown Chemical #1 | 2 |  |  |  |
| Unknown Chemical #1 | 3 |  |  |  |

| Unknown Chemical #2 | 1 |  |  |  |
| --- | --- | --- | --- | --- |
| Unknown Chemical #2 | 2 |  |  |  |
| Unknown Chemical #2 | 3 |  |  |  |

| Unknown Chemical #3 | 1 |  |  |  |
| --- | --- | --- | --- | --- |
| Unknown Chemical #3 | 2 |  |  |  |
| Unknown Chemical #3 | 3 |  |  |  |

| Unknown Chemical #4 | 1 |  |  |  |
| --- | --- | --- | --- | --- |
| Unknown Chemical #4 | 2 |  |  |  |
| Unknown Chemical #4 | 3 |  |  |  |

Data Calculations

- Convert your observed force applied to break in gf to forces in mN using the following conversion:

1 gf = 9.8 mN

- Record your values in a new data table with headers like below:

| mN Forces for Each Trial |  |  |
| --- | --- | --- |
| Trial | Measured weight to break (gf) | Calculated force to break (mN) |

- Using your converted forces calculate the surface tension measured with each trial using the following equation:

γ=FbreakC

where

- γ is the surface tension

- Fbreak is the force applied to break away from the liquid

- C is the circumference of your button in meters.

- Using your calculated surface tensions calculate the surface pressure of each chemical solution using the equation:

surface pressure=γ0-γA

where

- γ0 is the surface tension of pure water

- γA is the surface tension of the solution

Reflection Questions

- What chemical will you pursue as a potential surfactant? Support your answer with your collected data.

- What structural features could contribute to your chosen chemical as a potential surfactant? Be sure to include your chosen chemical’s structure here.