Building an Electric Circuit to Convert 
the Sensor Resistance into a Usable Voltage
INSTRUCTIONS

- Use this instruction manual to help you build an electric circuit to convert the sensor resistance into a usable voltage. The goal is to determine the relationships between applied force and sensor voltage output. Make sure you have the following items:

multimeter with sensor attached

11 6-inch wires (22 gauge)

breadboard

two 9V batteries

LM 741 op amp

30k Ω resistor

- Breadboard Basics

  - Letters on the breadboard are used to identify vertical columns.

Numb

Figure 2C

ers on the breadboard are used to identify horizontal rows.

  - The red lines in Figure 1 show how vertical columns and horizontal rows are internally connected.

  - When you connect a power source to a set of vertical columns, you can access voltage (+) and ground (-) from that set of vertically holes.

Figure 1

- Operational Amplifier (Op Amp) Basics

  - The main purpose of this integrated circuit is to amplify a weak signal.

  - It usually has two inputs (inverting input [2], non-inverting input [3]), and one output (6).

  - It has a wide range of functions: inverting amplifier, non-inverting amplifier, differential amplifier, voltage follower, summing amplifier, instrumentation amplifier.

  - In our circuit, we will use it as a non-inverting amplifier. See the Figure 2 schematic.

Figure 2

- Resistor Basics

  - As the name implies, a resistor is used in a circuit to create resistance to an electrical current. It limits the current passing through it.

  - Resistors have variable (Figure 3A) or fixed resistance (Figure 3B). A diagonal line through its symbol denotes that its resistance will vary.

  - The unit of a resistor is ohms (Ω).

Figure 3B

Figure 3A

- Op Amp 741 Placement

  - Locate pin 1, which is identified by a circle.

  - Place your op amp chip center in-between vertical column “e” and “f.”

  - Note: Do NOT place your chip in-between “a-e” or “f-j.”

Figure 4B

Figure 4A

- Op Amp 741 – Voltage Connection (V+)

Connect a wire from pin 7 to your voltage supply (vertical “+” column). See Figures 5A and 5B.

Figure 5B

Figure 5A

- Op Amp 741 – Ground Connection (V+)

Connect a wire from pin 4 to your ground supply (vertical “-“ column). See Figures 6A and 6B.

Figure 6B

Figure 6A

- Op Amp 741 – Non-Inverting Input Connection

Use 2 wires to connect from pin 3 to your ground supply (vertical “-“ column). See Figures 7A and 7B.

Figure 7B

Figure 7A

- Op Amp 741 – Inverting Input Connection

Connect a wire from pin 2 to any horizontal row above the op amp chip. See Figures 8A and 8B.

Figure 8B

Figure 8A

- Op Amp 741 – Output Connection

- Connect a wire from pin 6 to any horizontal row above the op amp chip.

- You can choose to horizontally align it to an inverting input row from the previous step.

- See Figures 9A and 9B.

Figure 9B

Figure 9A

- Resistor Connection – 30k Ohms

Place the 30k ohms resistor in-between your inverting input and output connections. 
See Figures 10A and 10B.

Figure 10B

Figure 10A

- Power Supply – Obtaining +9V and -9V

- Place the 9V batteries side by side, as shown in Figure 11A.

- Connect the two 9V batteries in series. To do this, connect the positive terminal of the left battery to the negative terminal of the right battery.

- Connect the multimeter to verify the output voltage on each side.

- Figure 11B shows a reading of a negative voltage.

- NOTE: REMEMBER WHICH SIDE OUTPUTS NEGATIVE and POSITIVE.

- NOTE: Connecting batteries incorrectly may cause the batteries to overheat!

Figure 11B

Figure 11A

- Power Supply – Power Op Amp with +9V

  - Disconnect wires in the previous step.

  - Connect the positive terminal to the positive vertical column on the breadboard.

  - Connect the negative terminal to the negative vertical column on the breadboard.

  - NOTE: Connecting batteries incorrectly may cause the batteries to overheat!

Figure 12

- Power Supply – Power Sensor with -9V

  - Connect the negative terminal to pin 1 of the FlexiForce sensor.

  - Connect a wire to pin 3 of the FlexiForce sensor

  - NOTE: PIN 2 DOES NOT GET USED!!

  - NOTE: Connecting batteries incorrectly may cause the batteries to overheat!

Figure 13B

Figure 13A

Figure 14

- Connect Sensor to Breadboard

  - Connect the positive terminal of the lower battery to the negative vertical column of the breadboard.

  - Connect pin 3 of the FlexiForce sensor to pin 2 of the op amp.

  - NOTE: Connecting batteries incorrectly may cause the batteries to overheat!