- Assembling the Sensor Handout

- Let’s get started constructing the Sharp GP2Y0A02YK0F “radar” system:

| 1. Sharp GP2Y0A02YK0F sensor | 2. four AA battery holder with cover & switch | 3. three AA alkaline batteries | 4. wire-connectors | 5. 18 AWG 2” shield wire |
| --- | --- | --- | --- | --- |
| 6. alligator clips | 7. #40 x ¾” machine screw | 8. #40 x ¾” (SAE) hex nut | 9. electrical tape | 10. class-shared wire stripper |
|  |  | 11. precision screwdriver set | Also: class-shared drill with a 9/64 drill bit | Also: class-shared Phillips screwdriver |

- Pre-Mark and Drill

  - Open the four AA battery holder using a Philips screwdriver.

  - Pre-mark the holder lid on the outside with two holes using a 9/64 drill bit about 37 mm apart. The holes should be directly above battery slot #1.

  - Verify that the battery holder does not contain batteries.

  - Identify the lower part of the four AA battery holder (not the lid).

  - Mark and drill into the side of slot #1 of the battery holder.

    - Pre-mark the outside lower part of the holder in the middle. Drill the hole using the same bit.

  - Mark and drill inside your holder (adjacent to the positive terminal of slot#1). This allows the holder’s VCC and ground wires to enter into slot #1.

    - Pre-mark the edge of the holder above VCC and ground wire opening with a 4 mm diameter. Drill the hole.

1b				     1e					1f

- Sensor Stabilization

  - Align the drilled holes on the holder lid with the sensor earlobe holes.

  - Run a #40 x 3/4” machine screw through each hole.

  - Secure the sensor by using a hex nut on each screw inside the battery holder.

2b					2c

- Serial and Parallel Holder Configuration

  - Correctly insert AA batteries into slots 2-4 (labeled as S2-S4).

  - Pop out the ground (negative) terminal in slot #1.

  - Make sure the shield wire is at least 2 inches long and stripped at each end.

    - Using two-inches of 18 AWG shield wire, connect it around the ground terminal.

  - Slide the ground terminal into slot #1 and slide the other end of the shield wire under the positive terminal. Use a multimeter to verify that you made a proper configuration.

3a				  3b				3c			         3d

- Sensor Connections

  - Run the ground (black) and VCC (red) wires coming out of the sensor unit through the hole on the battery holder lid (made in 1e).

  - Run the exiting ground (black) and VCC (red) wires from the lower part of the battery holder back inside to slot #1 (using hole made in 1f).

  - Group the ground (black) wire from the sensor and the holder along with an additional ground (black) wire inside the battery holder. Connect all three wires with a wire-connector. Run the third wire added out through the hole in the lower part of the battery holder (made in 1e).

  - Group VCC wires from the sensor unit and the battery holder. Connect them using a wire-connector.

  - Neatly place all connected components inside slot #1 of battery holder and close the battery holder.

4a			4c			4d		           4e

- Alligator Clips

  - Connect alligator clips to corresponding sensor output wire (yellow) and battery holder ground (black). Use a red alligator clip for the yellow wire and a black alligator clip for the black wire.

  - Use a bit of electrical tape to secure good connections.

5a					   5b