Young’s Modulus Practice Problems Answer Key

- A patient’s leg was put into traction, stretching the femur from a length of 0.46 m to 0.461 m. The femur has a diameter of 3.05 cm. With the knowledge that bone has a Young’s modulus of ~ 1.6  1010 in tension, what force was used to stretch the femur?

F= Y (∆L/L0)A

Y= 1.6 x 1010 Pa = 16000000000 Pa (given)

L0 = 0.46m (given)

ΔL = 0.001 m (obtained from .461 m - .46 m – the given amount of “stretch”)

A = πr2 = 3.14 (1.525)2 = 7.30 cm2 = 0.00073 m2 (obtained from given diameter of 3.05 cm)

Now, using the equation above:

F = 16000000000 x (.001/0.46) x 0.00073

F = 25,391.30 N

- Using the following information on stress and strain, plot a graph in Excel to determine the Young’s modulus for an unknown material. The radius of the material is 4 cm.

Cross-sectional area = A= πr2 = 50.24 cm2

| Initial Length (cm) | Final Length (cm) | Change in Length – ΔL (cm) | Strain (ΔL/L0) | Mass (g) | Force (N) | Stress (N/cm2) |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | 25.2 | 0.2 | 0.008 | 100 | 980 | 19.50637 |
| 25 | 25.7 | 0.7 | 0.028 | 200 | 1960 | 39.01274 |
| 25 | 26.3 | 1.3 | 0.052 | 300 | 2940 | 58.51911 |
| 25 | 26.9 | 1.9 | 0.076 | 400 | 3920 | 78.02548 |

Young’s modulus = 8.54 x 106 Pa